Khyber PakhtunKhwa Board of Technical Education Peshawar

Superintendent Sign:-

_________________

Roll No: - ___________

Candidate’s Sign:-

_____________________

D.A.E Model Paper

Ist Year Civil Technology (New Course)

Paper: - Survey – I (CT – 114)

-------------------------------------------------------------------------------------------------------------------------------

Time Allowed: - 	03 Hours. 							Max. Marks: - 100

Note: - 	Question No .1 is compulsory.

Time Allowed: - 	30 Minutes. 			( Part – A )			          	Marks: - 20

Q.1: - 	a)	Choose the correct answer.

- In chain Survey the area is divided into;

a) Rectangle 		b) Squares 		c) Triangles 		d) Circles

- The surface of still water is considered to be;

a) Level Surface 		b) Smooth surface 	c) Horizontal surface 	d) Slopping surface

- The difference between F.B and B.B of a line free from Local attraction is

a) 180°			b) 90°			c) 360°			d) 0°

- In accessible points in plane tabling are located by the method of .

a) Resection 			b) Traversing 		c) intersection		d) Radiation

- Current meter is used for the measurement of

a) Velocity 			b) Angle 		c) distance 		d) Area

b) 	Tick True / False.

- The length of engineering chain is 99´. 					True / False

- In the whole circle bearing system ; BB is = F.B + 180 			True/ False

- In leveling operations the H.I = B.M reading + B.S reading 		True / False

- Temporary adjustment consist of centering, leveling, focusing.  		True / False

- Plumbing fork is used in Hydrographic survey. 				True/ False

- Match the terms in Column A with appropriate words in column B and write down

your choice in the answer column.

| S.No | Answer | Column A | Mark | Column B |
| --- | --- | --- | --- | --- |
| 1 |  | Gunter’s chain | A | Height of instrument |
| 2 |  | Whole circle bearing | B | Sounding rod |
| 3 |  | Alidade | C | Plane tabling |
| 4 |  | Optical square | D | Prismatic compass |
| 5 |  | Hydrographic survey | E | 66 ft |

d) 	Fill in the blanks with suitable words.

- True bearing of a line is known as ____________________.

- A point of known elevation is known as _______________________.

- Contour line cross ridge lines at ________________________.

- The point at which location of instrument is changed is known as __________________.

- Radiation method is used in _____________________.

***************

Khyber PakhtunKhwa Board of Technical Education Peshawar

Diploma of Associate Engineer

Model Paper

Ist Year Civil Technology ( New Course )

Paper: - Survey – I (CT – 114)

-------------------------------------------------------------------------------------------------------------------------------

Time Allowed: - 	2 ½  Hrs. 						          Marks: - 80

( Part – B  )

Note: - 	Attempt any 11 questions. Each question carries 04 marks.

Q.1: - 		Compare the two systems of expressing the Magnetic bearings.

Q.2: - 		Define different types of leveling.

Q.3: - 		What do you understand by Orientation of a Plane Table.

Q.4: - 		How would you detect the presence of local attraction at a station?

Q.5: - 		What is the angular and linear check of a closed traverse?

Q.6: - 		What is the difference between Plane surveying and Geodetic Surveying?

Q.7: - 		Define bench marks and name the types of bench mark.

Q.8: - 		State the importance of a Datum surface in surveying.

Q.9: - 		Differentiate between contour interval and horizontal equivalent.

Q.10: - 	Define stadia constant.

Q.11: - 	Define hydrographic survey.

Q.12: - 	What do you understand by Sounding in hydrographic surveying?

Q.13: - 	Write primordial formula for computing the earth work in embankment.

Q.14: - 	Draw the cross section of a road fully in cutting.

Q.15: - 	Make a page of level book for recording the observed reading on ground.

( Part – C )

Note: - 	Attempt any three questions. All questions carry equal (12) marks.

Q.2: - 	 	The distance between two stations was measured by means of engineer’s. Chain which was

4 too short and the result was 1600 ft. Find the true distance between the two stations.

Q.3: - 		Name different methods of Plan Table survey and explain any one of them.

Q.4: - 	 	The following record refers to an operation involving reciprocal leveling. If the reduced

level of station A is 525.50, find the reduced level of station B.

| Instrument Station | Staff Reading |  | Remarks |
| --- | --- | --- | --- |
|  | A | B | Distance AB = 500m |
| A | 1.155 | 2.595 |  |
| B | 0.985 | 2.415 |  |

Q.5: -	 	Explain clearly the difference between Prismatic compass and Surveyor’s compass.

Q.6: - 	 	Describe the uses and characteristics of contour lines.

************************